| Tiertime UP300 printer Safe Operating Process |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This Safe Operating Process (SOP) is to be used in conjunction with any legislation, operating manuals, Australian or International Standards or task relevant training. |  |  |  |  |  |  |  |
| Description of the Work: |  |  |  |  |  |  |  |
| Operate the Tiertime UP300 printer in the Library Makerspace to complete 3D printing projects |  |  |  |  |  |  |  |
| Personal Protective Equipment Required: remove any which are not relevant |  |  |  |  |  |  |  |
|  | Safety glasses may be required for some operations |  |  |  |  | Safety gloves will be required when taking print models off the platform. |  |
|  | Door and lid of 3D printer should be closed when printer is in use |  |  |  |  | Long and loose hair must be contained. |  |
| Potential Hazards |  |  |  |  |  |  |
| - Fumes and particles |  |  |  |  | - Cuts when removing printed objects from the print board |  |
| - Burns from hot surfaces - especially the print nozzle and printhead |  |  |  |  | - Particles in eyes when trimming projects |  |
| - Pinching from the print fan |  |  |  |  | - Water leaks and electricity. Do not use near any wet surfaces |  |
| Pre-Operational Safety Checks: |  |  |  |  |  |  |
| - Make sure you have completed the 3D printer induction. - Open the UpStudio software. Make sure the “Sleep” setting is selected so that the machine will turn itself off after the print job is completed. - Print jobs must be completed before 4pm on the same day. - Check the Up300 is free from of obstructions that may prevent the machine from feeding material smoothly - Ensure the print board is correctly locked to the print table - Check that no maintenance tools are present in the UpBox - Ensure the spool and filament is not an entanglement risk - Check filament is loaded per instructions? Make sure you are familiar with how to change filament before doing so or ask a MakerSpace technician to assist - Ensure PLA filament is being used. We only support the use of PLA filament in the Library Makers. ABS filament is not to be used in the Library MakerSpace - Report any suspect equipment to your MakerSpace technician - Ensure you have the correct PPE - protective glasses, gloves |  |  |  |  |  |  |
| Operational Safety Checks: |  |  |  |  |  |  |
| - Turn on the Up300 Printer. - Ensure to initialise the printer selected via the UpStudio software. - Ensure you have elected the correct filament type setting - PLA setting - Close the Up300 door and lid during operations - Do not initiate movement of the print head whilst any foreign body including clothing, hair, fingers) is close to or entangled in the machine - Never touch the print nozzle, printhead and print bed during operations. This could result in burns or other personal injuries - Do not attempt to open the door whilst printer is in operation. - Never attempt to remove the nozzle from the print head - Never touch print head, print board and printed parts immediately after print operations as they will be HOT Especially the print head |  |  |  |  |  |  |
| Post-Operational Safety Checks: |  |  |  |  |  |  |
| - Turn off the printer before placing your hands in the printer and before removing parts - Take care when removing material from the 3D printer as the nozzle can be extremely hot - you will be required to wear gloves - Wear safety glasses and gloves when removing support material and trimming objects - Clear workspaces of any residue and debris and place waste in the appropriate bin |  |  |  |  |  |  |
| Housekeeping: |  |  |  |  |  |  |
| - Return PPE equipment to storage containers next to the 3D Printers - Clear workspaces of any residue and debris and place waste in the appropriate bin - Inform MakerSpace technicians of any problems or faults |  |  |  |  |  |  |
| Maintenance & Cleaning |  |  |  |  |  |  |
| Daily - Clear workspaces of any residue and debris and place waste in the appropriate bin - Clean print boards Weekly - Clean nozzle Other - Replace filter every 6 months or 300 hours of printing |  |  |  |  |  |  |
| Assessed by: |  | Charles Barnett |  |  |  |  |
| Date of Assessment: |  |  | 18 July 2018 |  |  |  |