| CARVEY CNC machine Safe Operating Process |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This Safe Operating Process (SOP) is to be used in conjunction with any legislation, operating manuals, Australian or International Standards or task relevant training. |  |  |  |  |  |  |  |
| Description of the Work: |  |  |  |  |  |  |  |
| Operate the Carvey CNC machine in the Library MakerSpace to complete projects. |  |  |  |  |  |  |  |
| Personal Protective Equipment Required: remove any which are not relevant |  |  |  |  |  |  |  |
|  | Safety glasses may be required for some operations. |  |  |  |  | Safety gloves may be required for some operations. |  |
|  | Safety guard / Carvey Lid must be used at all times. |  |  |  |  | Hearing protection may be required for some operations. |  |
|  | Face masks must be used at when opening the device. |  |  |  |  | Long and loose hair must be contained. |  |
| Potential Hazards |  |  |  |  |  |  |
| - Wet surfaces and electricity |  |  |  |  | - Fine particles when lid is opened after a project |  |
| - Using non approved materials could cause toxic fumes or damage the CNC machine - Absolutely no PVC (including vinyl), metal (including aluminium, or circuit boards), foam, treated or glued wood, ceramics, |  |  |  |  | - Cuts from sharp tool bits - Pinching hazard and finger jams form lid |  |
| Pre-Operational Safety Checks: |  |  |  |  |  |  |
| - Make sure you have completed the CARVEY induction. - Only use approved materials on the CARVEY. Obtain approval from a MakerSpace technician for use of any material not supplied by the MakerSpace. - Never attempt to cut magnesium or any other flammable material. Sparks or fire may occur if used. - Do not use Carvey with liquid materials or any materials containing liquid. - Do not attempt to cut anything bigger than the cutting bed - width (X-axis): 11.3 inches / 30 cm, Depth (Y-axis): 8 inches / 20 cm, Height (Z-access): 1.5 inches (3,81 cm) . - Ensure the material is clamped securely under the smart clamp and AT LEAST 2 other Carvey clamps. If you have trouble fastening your project ask a makerspace technician. - In Easel, ensure that there are no collisions with your model and clamps. The “step over height” machine setting under advanced can aid with this. - Use the ‘tab’ feature in easel to keep pieces secured during a cut-through operation. - Check that the cutting tool is in good condition (not burned, blunt or snapped). - Ensure that the collet is tightly secured and the correct cutting bit is installed. - Check that no tools are present in the Carvey cabinet. |  |  |  |  |  |  |
| Operational Safety Checks: |  |  |  |  |  |  |
| - Ensure the lid is closed (carvey should not operate if lid is is open) - Do not move the material or print bed during cutting operations. - Do not initiate movement of the cutting head whilst any foreign objects are close to or in machine (clothing, fingers hair). - Do not place fingers or tools inside the router while parts are cutting. |  |  |  |  |  |  |
| Post-Operational Safety Checks: |  |  |  |  |  |  |
| - Enrue you are wearing a face mask and glasses when you open the Carvey lid. Particles may be floating in the air. - Vacuum dust and debris BEFORE removing your part. - Do not move print bed or print head by hand. - Exercise caution when handling material around sharp cutting tools - especially when removing your project from CARVEY - Never touch tools immediately after cutting operations as they may be hot. - Do not use liquid or aerosol cleaners to clean the inside of Carvey’s work area. - Remove the tool/drill bit, clamps and leave the cabinet clean before leaving the MakerSpace. - Return any tools used in the project to the appropriate storage. |  |  |  |  |  |  |
| Housekeeping: |  |  |  |  |  |  |
| - Return PPE equipment to PPE storage |  |  |  |  |  |  |
| - Clear residue and debris from the workspace and place waste in appropriate bins |  |  |  |  |  |  |
| Maintenance & Cleaning |  |  |  |  |  |  |
| Daily / after each project - Vacuum inside the carvey - Clear carvey and workspace of any residue or debris and place waste in appropriate bins Weekly - Inspect and replace carving bits when necessary. (MS Technician) - Dust / wipe down the Carvey cabinet (MS Technician) Quarterly or as required - Clean under metal skid plates when necessary ( MS Technician ) - Replace waste board when necessary. (MS Technician ) - Tension or replace Carvey’s X and Y-Axis belts when they stretch or break. (MS Technician ) - Clean and service the smart clamp or replace if necessary. ( MS Technician ) |  |  |  |  |  |  |
| Assessed by: |  | Charles Barnett |  |  |  |  |
| Date of Assessment: |  |  | 18 July 2018 |  |  |  |